Reviewing the Architectural File

- Open the file MEP-GTSC-Architectural-M.

- View: Ground Floor Level.

- VV.

- Check the category: Plumbing Fixtures. OK.

- Select a toilet and check its type. These toilets are created by the architect.

- Close the file without saving changes.

Starting an MEP Project

- Models > New.

- Browse: Systems-Default_Metric Template. OK.

- Insert > Link Revit:

- Open the file MEP-GTSC-Architectural-M.

- Set the Positioning to Auto – Origin to Origin.

- Open.

- PIN.

- Review: Manage > Manage Links or right-click Revit Links in PB > Manage Links.

The Manage Links dialog has tabs for Revit models, IFC links, CAD Formats, DWF Markups, and Point Clouds. Under the tabs are columns that provide information about the link.

| Column | Description |
| --- | --- |
| Link Name | Indicates the name of the model or file being linked. |
| Status | Indicates whether the link is loaded in the host model. The field will display as Loaded, Not Loaded, or Not Found. An additional status, In Closed Workset, may display for a Revit link. When a Revit link is in a workset that is closed, the link is unloaded and does not display in model views. The link is still listed in the Manage Links dialog, and under Revit Links in the Project Browser. |
| Reference Type | Determines whether this linked model will be shown (Attachment) or hidden (Overlay) when the host model is linked to another model. This column applies to Revit and IFC models only. |
| Positions Not Saved | Indicates if the link's position is saved in the shared coordinate system. This column applies to Revit and CAD links. |
| Size | Size of the linked file. This column applies to CAD formats and DWF markups only. |
| Saved Path | Location of the link on your computer. In worksharing, this is the location of the central model. For Point Clouds, this path is relative to the Point Cloud Root Path. Click Options on the application menu to modify this root path. Note: If you modify the root path, you may need to reload any point cloud files that are already linked to the model. |
| Path Type | Indicates whether the link's saved path is relative, absolute, or the Revit Server path. This column does not apply to point clouds. |
| Local Alias | If you are using file-based worksharing and you have linked to a local copy of the Revit model or IFC model instead of linking to the central model, its location displays here. For example, you may want to link to a local model for improved performance when working with your local copy of the workshared model. When you Sync with Central, other users do not use your local copy of the linked model, nor do they see your local path. |

Modify the Display

- VV.

- Revit Links tab.

- Click by Host View.

- Click Basics tab.

- Select “By linked view”. Select Floor Plan: Ground Floor Level.

- OK, OK.

- Disable Select Links.

- Move the elevation markers (East, West,…) outside the linked file. This movement is reflected in the views for the current view discipline. If you activate other views in other disciplines, the elevation markers remain inside the linked file.

Copy and Monitor Levels

- Open the view: East – Mech.

- VV.

- Model Categories tab, turn off Plumbing Fixtures.

- In the Annotations tab, turn off Grids.

- OK.

- Modify drag the host levels (1 and 2) outside the linked file.

- Change elevation of Level 1 and Level 2 to match architectural linked levels.

- Collaborate > Copy/Monitor > Select Link.

- Click linked file.

- Click Monitor button.

- Click host Level 1 then linked Ground Floor Level.

- Repeat for Level 2 with linked Roof Deck Level.

- Finish.

Testing the Monitor Feature

- Select host Level 1.

- Change the Height to 1000mm.

- Read then close the warning message.

- Undo.

Copying the Remaining Levels

- Collaborate > Copy/Monitor > Select Link.

- Click the linked file.

- Click Options and check Levels tab.

- Make sure you choose “Reuse if Elements match exactly” for Reuse matching Levels parameter.

- Click Copy, check Multiple in option bar.

- CTRL select the top four levels.

- Check your selection by clicking on filter in option bar.

- Finish (next to Multiple).

- Go back to Copy/Monitor tab and click on Finish in the ribbon.

- Turn on the right bubbles for each of the four levels and turn off the left bubbles.

- Drag the bubbles of the new levels to the right to align them with Level 1 and 2.

Add and Monitor Levels

- Architecture > Level.

- Pick Lines.

- Uncheck Make Plan View in option bar.

- Set Offset to 600mm.

- Create a new level above level 1.

- Create another level above level 2.

- Unlock the level heads and pull them slightly outside.

- Rename the new levels: 
Level 1 – Reference
Level 2 – Reference

- Collaborate > Copy/Monitor > Select Link.

- Click the linked file.

- Click Monitor.

- Click Level 1 – Reference then the linked Ground Floor Level.

- Click Level 2 – Reference then the linked Roof Deck Level.

- Finish. If you forget Finish, you cannot perform other commands.

- Select “Level 2 – Reference” and change it’s elevation to 5100mm.

- Coordination Monitor alert message pops up in the bottom right corner.

- It shows that even if levels are not aligned but monitored, Revit will warn you.

- Close the warning message and undo.

Copy and Monitor MEP Fixtures

- View: 1 – Plumbing - VV and turn off Grids in Annotation Categories.

- Collaborate > Copy/Monitor > Select Link.

- Click the linked file.

- Click Coordination Settings.

- Select the Plumbing Fixtures category.

- Set the Copy Behavior to Allow batch copy.

- Set the Mapping behavior to Specify type mapping.

- Click Type Mapping under Plumbing Fixtures.

- In the Behavior area, you can change the Host Model type for each to any selected family from the host file (if you already loaded MEP families in your host file with the same shape of the architect’s).

In this specific case, we will use the original type from the linked architectural file and pipe connectors can be added later to the family.

- Leave the other categories to Copy Individually so they will not be copied to new type when using batch copy command.

- Click Save and Close. Clicking Copy would have copied all the fixtures not just the ones you mapped.

- Batch Copy.

- Select Copy the fixtures.

- Finish.

- Check all plumbing fixtures. They are in your host file now.

- Save the file as MEP-GTSC-Linking-M.

- Close the file.

Coordinate the Architectural and MEP Models

- Open the file MEP-GTSC-Architectural-M.

- View: East.

- Select Top Of Roof and change its name to Top Of Roof Floor.

- No to rename.

- Save and Close.

- Reopen MEP-GTSC-Linking-M.

- OK to close Warning.

- View: East – Mech => Level name of Top Of Roof of host file not updated.

- Collaborate > Coordination Review > Select Link.

- Click the linked file.

- Expand the warnings.

- From Action, select Rename Element.

- Click OK.

Note If the architect will be linking your MEP file then you should make sure in the Link Manager his file is set to Overlay. The Architect should delete his plumbing fixture now that you have recreated them and link your file as an Attachment.

Practice

- Open the file Bungalow.

- Change the level 2 height to 3400.

- Add a 600x600mm ceiling at 2600.

- Add 600x600 lighting fixtures using Systems > Lighting Fixtures, etc. Use Mechanical fixtures not architectural.

- Save and close the file.

- Create a new file based on the Systems template.

- Link the bungalow file.

- Replace the lighting fixtures with 600x1200 lighting fixtures.